Your code is a Bare-Metal Digital Signal Processing (DSP) application designed for the STM32L452 microcontroller. It takes raw mathematical data (signals) stored in the chip's memory and "streams" it to your computer so you can visualize it as a graph.

Here is the technical breakdown of the most important aspects:

1. Hardware Initialization (The "Foundation")

- FPU Enable (fpu_enable): Your chip has a dedicated hardware "Math Engine" called the Floating Point Unit (FPU). By default, it is turned off to save power. Your code manually enables "Full Access" to this unit so the processor can handle decimal numbers (floats) instantly instead of using slow software calculations.

- UART Driver (uart_tx_init): This sets up the "Serial Port." It talks directly to the hardware registers to tell the chip which physical pin to use (PA2), what speed to talk at (115200 baud), and to turn on the transmitter.

- Register Polling: In uart2_write, you use a while loop to "poll" the Status Register (ISR). This makes the CPU wait until the previous character is physically sent out before trying to cram the next one into the Transmit Data Register (TDR).

2. Standard I/O Redirection (The "Bridge")

- printf Retargeting: Normally, printf tries to print to a screen you don't have. You implemented a special function called __io_putchar. Because of this, every time you call printf, the code "diverts" that text into your uart2_write function, sending it across the USB cable to your PC.

3. Data Streaming Logic

- The Plotting Loop: Functions like serial_plot_input_sig act as a "Data Pump." They iterate through a massive array of pre-calculated numbers and send them one by one.

- Pseudo Delay: You use a "Burn" loop (pseudo_dly) to waste time. This prevents the STM32 from sending the data too fast for the Arduino Serial Plotter to keep up.

Understanding the Signal Naming Convention

The names in your signals.h and signals.c files follow standard Digital Signal Processing (DSP) naming rules:

- f32: This stands for 32-bit Floating Point. It tells you that every number in that array is a decimal (e.g., 0.592465), not a whole integer.

- _5hz_signal: This describes the Frequency. "5Hz" means if you plotted this signal over 1 second, the wave would repeat itself exactly 5 times.

- 1kHz_15kHz: This indicates a Composite Signal. It means the data is a mixture of a low-pitch 1,000Hz wave and a high-pitch 15,000Hz wave added together.

- impulse_response: In DSP, this is often called the Kernel. It represents a mathematical "Filter." If you were to "convolve" (mix) your input signal with this impulse response, it would remove the high-pitch 15kHz noise and leave you with just the clean 1kHz signal.

- SIG_LEN / LENGTH: These constants tell the computer the Buffer Size (how many items are in the list) so it doesn't try to read past the end of the memory.